Materials:

- (1) MQ-3 Sensor

- (1) Nexys A7-T100 FPGA

- (1) LauchPad (TM4C123GH6PM)

- (1) breadboard

- (1) Potentiometer (1Kohms)

- (1) 2kOhm resistor

- Jumper cables

Start-up:

- This sensor requires a warm-up period in order to obtain the best readings. Connect the sensor’s VCC and Ground pins to the corresponding VBUS and Ground pins on the Launchpad. Turn on the Launchpad and allow the sensor to warm up for at least 15-30 minutes. The sensor does not need to be connected to the FPGA yet, but should be monitored either by oscilloscope or voltmeter to ensure the monitor level has dropped.

Vivado Code:

XADC Set-up:

- Go to the IP Catalog and search for the XADC Wizard.

- Give a component name since different sensors will have different specifications. (Ex: MQ3_XADC)

- Configurations:

  - Basic tab:

    - Interface Option: Choose DRP (dynamic reconfiguration port). Purpose is that the hardware is controlled by an implemented state machine and not another processor.

    - Startup Channel Selection: Choose Channel Sequencer. This allows for the exact analog channels the FPGA should pay attention to. For this demo, I am asking to pay attending to vauxp3/vauxn3 which are the right most pins on the JXADC header.

    - Timing Mode: Choose Continuous Mode since we are live sampling at a continuous duration and outputting live values.

    - LEAVE ALL OTHER CONFIGURATIONS THE SAME.

  - ADC Setup Tab

    - Sequencer Mode: choose Continuous

    - Channel Averaging: choose None

    - ADC Calibration: check only ADC Offset and Gain Calibration

    - Supply Sensor Calibration: check only Sensor Offset and Gain Calibration

    - Check Enable CALIBRATION Averaging

    - LEAVE ALL OTHERS UNCHECKED

  - Alarms: I left these configurations at default. These can be programmed to LEDs or other logic for ensure safe use.

    - User temperature alarm

      - 60 temp reset

      - 85 temp trigger

    - VCCINT Alarm

      - 0.97-1.03V

    - VCCAUX Alarm

      - Range: 1.75-1.89 V

  - Channel Sequencer

    - Only check VAUXP3/VAUXN3 (The pins I am connecting to the sensor)

- Click OK and then Generate leaving the Out-of-Context IP checked

- Check if the .xci file is found in the design source section. If found, then the IP source is ready to be used in the top module.

Writing the Top Module

Now that have IP source to program the XADC core, we can begin to write the logic for sampling from the analog signal to be converted into a digital value. The main components for this code are:

- Component instantiation of the xadc_mq3_sensor (or what you called your IP core)

  - A template can be retrieved from the IP source under instantiation template

- A process for controlling the movement of the state machine and an asynchronous reset

- The state machine that controls sampling process from the XADC module

  - When IDLE,

    - Waits for end of sequence flag to enable the reading

    - Otherwise, stays IDLE

  - When WAIT_DRDY,

    - Wait for the xadc_ready flag indicating the analog signal sampled has been converted to a digital signal

  - When SAVE_DATA,

    - This state saves the data in another process

  - Otherwise,  stay in the IDLE state

- Save data process

  - This process reads from the do_out register when in the SAVE_DATA state

- A statement to display values from the adc_data vector

  - You can read the 12-bit data and output onto the LED

  - Optional: I am currently using a leveled approach where LEDS turn on according to a certain alcohol level. This was used only to verify that the FPGA was correcting reading the analog signal.